Scholars International Journal of Traditional and Complementary Medicine

Abbreviated Key Title: Sch Int J Tradit Complement Med

ISSN 2616-8634 (Print) |ISSN 2617-3891 (Online)

Scholars Middle East Publishers, Dubai, United Arab Emirates

Journal homepage: https://saudijournals.com

|  | Original Research Article |
| --- | --- |

Method Development and Validation for Detection of Selective Elemental Impurities in Clarithromycin API by ICP-MS

Kamalathiyagarajan D1, Sekar V1*, Krishnan R1, Mohanapriya N1, Venkatesan M1

1Department of Pharmaceutical Analysis, J. K. K. Nattraja College of Pharmacy, Kumarapalayam-638613, Tamil Nadu, India

DOI: https://doi.org/10.36348/sijtcm.2026.v09i01.001	   | Received: 16.09.2025 | Accepted: 11.11.2025 | Published: 21.01.2026

*Corresponding author: Sekar V

Department of Pharmaceutical Analysis, J. K. K. Nattraja College of Pharmacy, Kumarapalayam-638613, Tamil Nadu, India

| Abstract |  |
| --- | --- |

The present study focuses on the method development and validation for detection of selective elemental impurities in Clarithromycin active pharmaceutical ingredient (API) using Inductively Coupled Plasma–Mass Spectrometry (ICP-MS). Elemental impurities, originating from catalysts, raw materials, or manufacturing processes, can adversely affect product safety and efficacy. Hence, a sensitive and precise analytical method was developed in accordance with ICH Q3D, USP <232>, and <233> guidelines. The optimized ICP-MS parameters ensured accurate quantification of 23 elemental impurities, including Cd, Pb, As, Hg, Co, Ni, V, Pd, Pt, Ir, Rh, Ru, Se, Ag, Mo, Sb, Cu, Sn, Cr, Ba, Li, Tl, and Au, in a single analytical run without the need for internal standards. The developed method was validated for linearity, accuracy, precision, specificity, limit of detection (LOD), and limit of quantification (LOQ). The method exhibited strong linear correlation (r² ≥ 0.99), recovery within 70–150%, and relative standard deviation (RSD) <20%, satisfying USP acceptance criteria. The Kinetic Energy Discrimination (KED) mode minimized polyatomic interference, improving signal stability and reducing analysis time. This validated method proved to be simple, rapid, rugged, cost-effective, and suitable for routine quality control analysis of Clarithromycin API to ensure compliance with elemental impurity regulations and pharmaceutical safety standards.

Keywords: Clarithromycin, Elemental Impurities, ICP-MS, ICH Q3D, USP <232>/<233>, Method Validation.

Copyright © 2026 The Author(s): This is an open-access article distributed under the terms of the Creative Commons Attribution 4.0 International License (CC BY-NC 4.0) which permits unrestricted use, distribution, and reproduction in any medium for non-commercial use provided the original author and source are credited.

INTRODUCTION

Analytical chemistry is a fundamental branch of chemistry concerned with the qualitative and quantitative determination of chemical substances. It plays a pivotal role in research, development, and quality control across pharmaceutical, environmental, biological, and industrial applications. The discipline encompasses several key domains, including chemometrics, forensics, bioanalysis, clinical analysis, environmental monitoring, and materials characterization. Analytical chemistry employs both classical and instrumental techniques to ensure the accuracy, reliability, and reproducibility of results in scientific investigations.

Instrumental methods form the cornerstone of modern analytical procedures. These include conductometry, potentiometry, polarography, UV-Visible spectroscopy, infrared spectroscopy, fluorimetry, atomic absorption and emission spectrometry, flame photometry, mass spectrometry, nuclear magnetic resonance (NMR), thin-layer chromatography (TLC), high-performance thin-layer chromatography (HPTLC), high-performance liquid chromatography (HPLC), and gas chromatography (GC). Among these, mass spectrometry (MS) has emerged as one of the most effective techniques for molecular structure elucidation, as it provides detailed information about the mass-to-charge (m/z) ratios of ions formed during ionization, which helps in identifying molecular structures and bond characteristics.

A specialized and highly sensitive form of mass spectrometry, Inductively Coupled Plasma Mass Spectrometry (ICP-MS), is widely used for the detection and quantification of trace and ultra-trace elements. Developed by Grey and Houk in 1980 and commercialized by Sciex in 1983, ICP-MS revolutionized elemental analysis by combining plasma-based ionization with mass spectrometric detection. It offers outstanding sensitivity, wide linear dynamic range (up to nine orders of magnitude), and the capability to simultaneously detect more than 70 elements at concentrations as low as parts per trillion (ppt).

In ICP-MS, the sample is first converted into an aerosol and introduced into an argon plasma, where it is atomized and ionized at temperatures reaching 10,000 K. The generated ions are directed through a series of electrostatic lenses, known as ion optics, and then separated based on their m/z ratio by a quadrupole mass analyzer. Finally, the ions are detected and quantified, providing precise elemental concentrations. The key components of an ICP-MS include the nebulizer, spray chamber, plasma torch, interface cones, ion optics, quadrupole mass analyzer, and detector.

ICP-MS offers several advantages over traditional techniques such as Atomic Absorption Spectroscopy (AAS) and Inductively Coupled Plasma Atomic Emission Spectroscopy (ICP-AES), including multi-elemental detection, high sample throughput, and superior isotope ratio measurement. Analytical interferences such as isobaric overlaps, polyatomic and doubly charged ions, and space-charge effects may affect accuracy, but these are effectively mitigated using collision/reaction cell (CRC) systems and kinetic energy discrimination (KED) technology. Moreover, microwave-assisted acid digestion is employed for solid or complex samples to ensure complete dissolution and minimize contamination prior to analysis.

Given its precision, sensitivity, and selectivity, ICP-MS is now a preferred technique in regulatory, pharmaceutical, and environmental laboratories. It is particularly valuable in detecting elemental impurities in pharmaceuticals, as per ICH Q3D and USP <232>/<233> guidelines, ensuring drug safety and compliance with international regulatory standards.

METHOD VALIDATION

Method validation is an essential process in analytical chemistry that confirms whether a developed analytical method is suitable for its intended purpose. It ensures that the method consistently produces accurate, reliable, and reproducible results under defined conditions. According to the International Council for Harmonisation (ICH) and United States Pharmacopeia (USP) guidelines, validation is a critical component of quality assurance in pharmaceutical analysis.

Analytical methods typically requiring validation include:

- Identification tests to confirm the presence of a specific analyte.

- Quantitative tests for impurities to determine impurity levels in substances.

- Limit tests for impurity control to ensure impurities remain within acceptable limits.

- Quantitative assays for the active pharmaceutical ingredient (API) or other critical components in a drug product.

The choice of validation parameters depends on the purpose of the analytical method. The commonly evaluated validation characteristics include specificity, accuracy, precision, repeatability, intermediate precision, linearity, range, detection limit (LOD), quantitation limit (LOQ), and robustness.

- Specificity ensures the method can accurately measure the analyte without interference from other substances.

- Accuracy measures the closeness between the experimental result and the true value.

- Precision assesses the degree of variation among replicate measurements and includes repeatability (intra-day precision) and intermediate precision (inter-day or analyst-to-analyst variation).

- Linearity and range determine the proportionality between analyte concentration and response within a defined interval.

- Detection limit (LOD) and quantitation limit (LOQ) establish the smallest detectable and quantifiable concentrations, respectively.

- Robustness evaluates the method’s resilience to small, deliberate variations in analytical conditions, such as pH, temperature, and flow rate.

AIM AND OBJECTIVE

The term "Quality" of a drug product is defined as the sum of all aspects that contribute either directly or indirectly to the drug's effectiveness and safety.

The literature review indicated that most analytical methods were developed based on other analytical techniques such as UV, HPLC, etc. Except for ICPMS in selected molecules.

- Clarithromycin

As a result, the focus of our research has been on the development and validation of analytical methods for selected molecules utilising ICP-MS technology.

The aim and objective of the proposed work is

- Determine and optimize parameters such as plasma power, gas flows, and sample introduction settings to enhance sensitivity and stability.

- To develop an analytical method with acceptable sensitivity, accuracy, precision, linearity, specificity, and ruggedness for the estimation of elemental impurities in selected drug molecules by using ICP-MS

- To perform the validation for the developed method as per Q2R1, Q3D, and USP guidelines (232 and 233).

- To develop an analytical method that is suitable for routine analysis in quality control laboratories.

Plan of Work

- Review of literature

- Selection of drug molecules

- Choice of solvent

- Determination of solubility properties

- Selection of options as per ICH Q3D

- Optimisation of ICP-MS and Digester Conditions

- Validation of the method as per ICH and USP guidelines

- Analysis of pharmaceutical drug substances

- Application of the proposed method of estimation of pharmaceutical drug substances

MATERIALS AND METHODS

Table 01: List of Instruments

| Name of the instruments | Manufacturer/Supplier |
| --- | --- |
| Digital Micropippete | Eppendorf |
| Digital balance | Sartorius |
| Bath sonicator | Sartorius |
| Digester | Titan MPS |
| ICP-MS | Nexion 5000 controlled by Syngistics 2.0 |

Table 02: List of Chemicals, Reagents and Materials

| Chemical/Reagent Materials | Suppliers |
| --- | --- |
| Elementals free grade Water | Milli-Q grade 18 micro ohm |
| Nitric acid (Elemental impurities free grade) | Fisher scientific |
| Hydrochloric acid (Elemental impurities free grade) | Fisher scientific |
| Hydrogen peroxide (Elemental impurities free grade) | Fisher scientific |

Table 03: List of Active Pharmaceutical Ingredients and Standards

| API & Standard Materials | Suppliers |
| --- | --- |
| Clarithromycin | P&N Pharma and neutraceuticals, Chennai |
| Elemental impurity standards | Inorganic venture |

Instrument Parameters - ICPMS Sampling Devices

Peristaltic pump control: No, Sample flush time: 150, Sample flush speed: 50, Read delay time: 50 sec

Read delay and analysis speed: 20.00, Wash time: 180, Wash speed: 45.00

Timing Parameters

Sweep reading: 40, Number of replicates: 3, Dwell time: 60.0 ms

Signal Processing

Detector Mode: Dual, Measurements unit: cps, QID: On,

Plasma Condition

RF Power: 1700W, Carrier gas flow: 1.02 L/min, Mode: Helium KED, He flow: On, He Flow rate: 4.5ml/min

Method Development – Clarithromycin Api

Fixing of Permitted Concentration of Elemental Impurities (µg/g)

Cadmium Oral PDE 5 µg/Day

5 µg/Day

Concentration µg/g =------------------------ = 0.5 µg/gm

10 gm /Day

Target limit or Target concentration:

The acceptance value for the elemental impurity being evaluated. Exceeding the Target limit indicates that a material under test exceeds the acceptable value.

J: The concentration (w/w) of the element(s) of interest at the Target limit, appropriately diluted to the working range of the instrument

PDE µg/Day

J Target Concentration = Maximum Daily dose g/ Day X Dilution factor Dilution factor: 0.05 g of sample into 25 ml diluent

Method Development Trails

Table 04: Digester Condition -01

| Temperature (°C) | Pressure(bar) | Ramp | Hold time (Min) | Power (%) |
| --- | --- | --- | --- | --- |
| 50 | 35 | 5 | 5 | 80 |
| 70 | 35 | 5 | 5 | 80 |
| 150 | 35 | 5 | 15 | 90 |
| 50 | 35 | 5 | 5 | 0 |

Table 05: Different Solvent Ratio for method development process

| S. No | HNO3 (ml) | HCl (ml) | H2O2 (ml) |
| --- | --- | --- | --- |
| 01 | 2.5 | 0.5 | - |
| 02 | 3.5 | 0.5 | 0.2 |
| 03 | 4.5 | 1.0 | 0.6 |

Table 06: Method Development trails Results for Accuracy

| S. No | Elements | HNO3: HCl (2.5 ml: 0.5 ml) | HNO3 : HCl : H2O2 (3.5 ml: 0.5 ml : 0.2 ml) | HNO3 : HCl : H2O2 (4.5 ml: 1 ml : 0.6 ml) |
| --- | --- | --- | --- | --- |
| 1 | Li | 85.11 | 89.21 | 95.90 |
| 2 | V | 75.61 | 85.54 | 98.88 |
| 3 | Cr | 85.22 | 91.65 | 97.56 |
| 4 | Co | 85.11 | 91.12 | 99.53 |
| 5 | Ni | 91.74 | 93.12 | 99.27 |
| 6 | Cu | 85.15 | 91.12 | 96.98 |
| 7 | As | 89.11 | 91.42 | 93.11 |
| 8 | Se | 76.11 | 82.73 | 98.54 |
| 9 | Mo | 83.61 | 87.15 | 97.08 |
| 10 | Ru | 75.09 | 89.51 | 99.11 |
| 11 | Rh | 78.11 | 87.02 | 98.56 |
| 12 | Pd | 76.12 | 88.12 | 99.84 |
| 13 | Ag | 75.98 | 88.29 | 99.43 |
| 14 | Cd | 91.23 | 93.11 | 99.87 |
| 15 | Sn | 92.10 | 97.54 | 102.36 |
| 16 | Sb | 94.78 | 98.12 | 105.97 |
| 17 | Ba | 91.33 | 95.14 | 100.02 |
| 19 | Ir | 92.63 | 97.33 | 102.23 |
| 20 | Os | 155.09 | - | - |
| 20 | Pt | 89.14 | 94.17 | 98.76 |
| 21 | Au | 91.42 | 93.12 | 98.34 |
| 22 | Hg | 87.32 | 96.67 | 99.90 |
| 23 | TI | 89.99 | 95.63 | 101.12 |
| 24 | Pb | 79.27 | 87.98 | 98.96 |

Observation:

Solvent Combination 1: (2 ml HNO3:0.5 HCl), recovery results were not satisfactory. Due to the formation of Osmium tetroxide, recovery range for Osmium exceeded the acceptance criteria.

Solvent Combination 2: (3.5 ml HNO3: 0.5 ml HCl: 0.2 ml H2O2)

Nitric acid ratio increased and with the addition of hydrogen peroxide. Recovery results found in satisfactory level except selenium.

Solvent Combination 3: (4.5 ml HNO3: 1 ml HCl: 0.6 ml H2O2)

To adjust selenium recovery result, nitric acid and hydrogen peroxide solvents volume increased.

All the results were found within acceptance criteria. Hence solvent combination 3 finalized for validation. Hydrogen peroxide additional advantages are that there is no acid corrosion of the digestion vessel PTFE walls, no formation of insoluble salts with an acid anion, and no change of the sample matrix by an acid. Because of its strong oxidation power, only small amounts of H2O2 need be used so concentrated sample solutions can be obtained.

Table 07: Optimized Digester Condition

| Temperature (°C) | Pressure(bar) | Ramp | Hold time | Power (%) |
| --- | --- | --- | --- | --- |
| 50 | 35 | 5 | 10 | 80 |
| 100 | 35 | 5 | 10 | 80 |
| 180 | 35 | 5 | 15 | 90 |
| 50 | 35 | 5 | 5 | 0 |

After digestion, samples were transferred into a 25 ml polypropylene tubes. Made up to the volume 25 ml with water.

Method Validation Parameters

Linearity / System Suitability Test (SST)

A system suitability test (SST) was performed to verify the adequacy and performance of the analytical method prior to validation. This test ensures that the system, instrument, and analytical procedure function properly to produce accurate and reliable results.

Linearity was established by analyzing a series of standard solutions at different concentration levels, ranging from 25% (LOQ level) to 200% of the target concentration. The calibration curve for each element was plotted between concentration and instrument response, and the correlation coefficient (r²), slope, and intercept were calculated to assess linearity.

Standard Stock Solution Concentrations (Except Osmium): Cadmium – 5 ppm; Lead – 5 ppm; Arsenic – 15 ppm; Mercury – 30 ppm; Cobalt – 50 ppm; Vanadium – 100 ppm; Nickel – 200 ppm; Thallium – 8 ppm; Gold – 100 ppm; Palladium – 100 ppm; Iridium – 100 ppm; Rhodium – 100 ppm; Ruthenium – 100 ppm; Selenium – 150 ppm; Silver – 150 ppm; Platinum – 100 ppm; Lithium – 55 ppm; Antimony – 120 ppm; Barium – 140 ppm; Copper – 300 ppm; Molybdenum – 300 ppm; Tin – 600 ppm; Chromium – 1100 ppm.

Standard Solution Preparation (Except Osmium):

From each stock solution, 1.0 mL was pipetted into a volumetric flask and diluted to 50 mL with the diluent. From this solution, 0.125 mL, 0.250 mL, 0.500 mL, 0.750 mL, and 1.000 mL were individually transferred into 50 mL PTFE tubes and further diluted to volume with the same diluent to obtain linearity levels corresponding to 25%, 50%, 100%, 150%, and 200%.

The obtained calibration data demonstrated excellent linearity within the specified range, confirming the suitability of the ICP-MS system for quantitative analysis of elemental impurities.

Table 08: Linearity Standard Concentration

| S. No | Elements | Intermediate Concentration (ppb) | 0.25 J (ppb) | 0.50 J (ppb) | 1.00J (ppb) | 1.50 J (ppb) | 2.00 J (ppb) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Cd | 100 | 0.250 | 0.500 | 1.000 | 1.500 | 2.000 |
| 2 | Pb | 100 | 0.250 | 0.500 | 1.000 | 1.500 | 2.000 |
| 3 | As | 300 | 0.750 | 1.500 | 3.000 | 4.500 | 6.000 |
| 4 | Hg | 600 | 1.500 | 3.000 | 6.000 | 9.000 | 12.000 |
| 5 | Co | 1000 | 2.500 | 5.000 | 10.000 | 15.000 | 20.000 |
| 6 | V | 2000 | 5.000 | 10.000 | 20.000 | 30.000 | 40.000 |
| 7 | Ni | 4000 | 10.000 | 20.000 | 40.000 | 60.000 | 80.000 |
| 8 | Tl | 160 | 0.400 | 0.800 | 1.600 | 2.400 | 3.200 |
| 9 | Au | 2000 | 5.000 | 10.000 | 20.000 | 30.000 | 40.000 |
| 10 | Pd | 2000 | 5.000 | 10.000 | 20.000 | 30.000 | 40.000 |
| 11 | Ir | 2000 | 5.000 | 10.000 | 20.000 | 30.000 | 40.000 |
| 12 | Rh | 2000 | 5.000 | 10.000 | 20.000 | 30.000 | 40.000 |
| 13 | Ru | 2000 | 5.000 | 10.000 | 20.000 | 30.000 | 40.000 |
| 14 | Se | 2000 | 5.000 | 10.000 | 20.000 | 30.000 | 40.000 |
| 15 | Ag | 3000 | 7.500 | 15.000 | 30.000 | 45.000 | 60.000 |
| 16 | Pt | 3000 | 7.500 | 15.000 | 30.000 | 45.000 | 60.000 |
| 17 | Li | 2000 | 5.000 | 10.000 | 20.000 | 30.000 | 40.000 |
| 18 | Sb | 24000 | 60.000 | 120.000 | 240.000 | 360.000 | 480.000 |
| 19 | Ba | 28000 | 70.000 | 140.000 | 280.000 | 420.000 | 560.000 |
| 20 | Cu | 60000 | 150.000 | 300.000 | 600.000 | 900.000 | 1200.000 |
| 21 | Mo | 60000 | 150.000 | 300.000 | 600.000 | 900.000 | 1200.000 |
| 22 | Sn | 120000 | 300.000 | 600.000 | 1200.000 | 1800.000 | 2400.000 |
| 23 | Cr | 220000 | 550.000 | 1100.000 | 2200.000 | 3300.000 | 4400.000 |

Precision

Precision is a key parameter in analytical method validation that reflects the degree of agreement among individual test results obtained under prescribed conditions. It demonstrates the reproducibility of the analytical procedure when applied repeatedly to multiple aliquots of a homogeneous sample.

For this study, six individual samples were prepared and analyzed at the 100% specification level to evaluate method precision. Each sample was spiked with the standard solution as per the specified concentration for the target elemental impurities.

Sample Preparation for Precision (Except Osmium):

Accurately weighed 0.05 g of the sample was transferred into separate clean, dry PTFE digestion vessels. To each vessel, 4.5 mL of nitric acid (HNO₃), 1.0 mL of hydrochloric acid (HCl), and 0.6 mL of hydrogen peroxide (H₂O₂) were added. The respective standard solutions were then spiked at 100% of the target concentration. A sample blank (without API and standard) was also prepared and analyzed simultaneously to ensure method specificity.

The digestion vessels were allowed to stand for 5 minutes at room temperature for pre-digestion, facilitating initial reaction and gas release. Following this, the vessels were sealed and loaded into the microwave digester and processed according to the optimized digestion program parameters.

After digestion, the clear solutions were transferred to volumetric flasks, diluted to the required volume with diluent, and subjected to ICP-MS analysis. The precision of the method was evaluated based on the % Relative Standard Deviation (%RSD) of replicate measurements, which was found to be within the acceptable limit (≤ 20%), confirming the repeatability and reliability of the developed analytical method.

Table 09: Digester condition -Precision

| Temperature (°C) | Pressure(bar) | Ramp | Hold time (Min) | Power (%) |
| --- | --- | --- | --- | --- |
| 50 | 35 | 5 | 10 | 80 |
| 100 | 35 | 5 | 10 | 80 |
| 180 | 35 | 5 | 15 | 90 |
| 50 | 35 | 5 | 5 | 0 |

After digestion, samples were transferred into a 25 ml polypropylene tubes Made up to the volume 25 ml with water.

Accuracy

Accuracy represents the closeness of agreement between the value obtained by the analytical method and the true or accepted reference value. The accuracy of the developed method was evaluated by recovery studies conducted at three concentration levels — 25%, 50%, 100%, and 150% of the target concentration.

Standard Stock Solution Concentrations (Except Osmium):

Cadmium – 5 ppm; Lead – 5 ppm; Arsenic – 15 ppm; Mercury – 30 ppm; Cobalt – 50 ppm; Vanadium – 100 ppm; Nickel – 200 ppm; Thallium – 8 ppm; Gold – 100 ppm; Palladium – 100 ppm; Iridium – 100 ppm; Rhodium – 100 ppm; Ruthenium – 100 ppm; Selenium – 150 ppm; Silver – 150 ppm; Platinum – 100 ppm; Lithium – 55 ppm; Antimony – 120 ppm; Barium – 140 ppm; Copper – 300 ppm; Molybdenum – 300 ppm; Tin – 600 ppm; Chromium – 1100 ppm.

Preparation of Accuracy Standards (Except Osmium):

Aliquots of 0.125 mL, 0.250 mL, 0.500 mL, and 0.750 mL from the 1 mL stock (diluted to 50 mL) were transferred into individual 50 mL PTFE tubes and diluted to volume to obtain 25%, 50%, 100%, and 150% levels, respectively.

Accuracy Sample Preparation:

Accurately weighed 0.05 g of sample was transferred into a clean PTFE digestion vessel. To this, 4.5 mL of nitric acid, 1.0 mL of hydrochloric acid, and 0.6 mL of hydrogen peroxide were added. Known amounts of standards were spiked into the sample at 25%, 50%, 100%, and 150% levels. Blank (without API) and control (without spike) samples were also prepared. The mixtures were allowed to stand for 5 minutes for pre-digestion, then digested under the optimized program.

The percentage recovery for each element was calculated and found within the acceptable range of 70–150%, confirming the accuracy of the developed ICP-MS method.

Table 10: Digester Condition - Accuracy

| Temperature (°C) | Pressure(bar) | Ramp | Hold time (Min) | Power (%) |
| --- | --- | --- | --- | --- |
| 50 | 35 | 5 | 10 | 80 |
| 100 | 35 | 5 | 10 | 80 |
| 180 | 35 | 5 | 15 | 90 |
| 50 | 35 | 5 | 5 | 0 |

LOQ Precision

The Limit of Quantitation (LOQ) is the lowest concentration of an analyte that can be quantified with acceptable precision. LOQ-level (25% of specification) standards were prepared and injected six times to assess precision. The %RSD values obtained were within acceptable limits, confirming method consistency at the LOQ level.

Limit of Detection (LOD)

The Limit of Detection (LOD) is the minimum concentration of an analyte that can be reliably detected but not necessarily quantified. LOD-level standards were prepared by serial dilution and injected in triplicate. The method demonstrated adequate sensitivity, confirming its suitability for trace-level elemental analysis.

RESULTS AND DISCUSSION

Results for Clarithromycin

Linearity/SST

      - Prepared Standard solutions in different concentrations From LOQ level (25%) to 200 % as per the specification limits. Intercept and Slope calculated for each elements from linearity curve. The correlation co-efficient should not be less than 0.99

Table 10: Linearity results of elemental Impurities in Intensity (cps)

| S. No. | Elements | 25 % | 50% | 100% | 150% | 200% |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Cd | 532.46 | 1070.49 | 2246.54 | 3222.60 | 4298.65 |
| 2 | Pb | 6029.61 | 10985.45 | 22077.12 | 33168.80 | 44260.47 |
| 3 | As | 280.52 | 608.35 | 1198.02 | 1809.69 | 2351.36 |
| 4 | Hg | 8548.30 | 15118.92 | 30260.14 | 45401.37 | 60542.59 |
| 5 | Co | 23250.17 | 46509.92 | 93029.40 | 139548.89 | 186068.37 |
| 6 | V | 18694.79 | 43091.13 | 76883.82 | 115676.51 | 154469.2 |
| 7 | Ni | 28313.69 | 48659.50 | 97351.11 | 146042.73 | 194734.34 |
| 8 | Tl | 22834.27 | 43680.10 | 87371.76 | 131063.42 | 174755.08 |
| 9 | Au | 112515.05 | 245095.99 | 450257.87 | 675419.75 | 900581.63 |
| 10 | Pd | 38806.55 | 77619.78 | 155246.22 | 232872.67 | 310499.11 |
| 11 | Ir | 228069.29 | 459226.44 | 921540.75 | 1383855.06 | 1846169.37 |
| 12 | Rh | 133917.13 | 268249.94 | 536915.54 | 805581.15 | 1074246.75 |
| 13 | Ru | 61400.07 | 122811.22 | 245633.53 | 368455.84 | 491278.15 |
| 14 | Se | 205.44 | 419.22 | 846.78 | 1254.34 | 1701.9 |
| 15 | Ag | 119367.17 | 242823.97 | 425737.58 | 638651.19 | 851564.8 |
| 16 | Pt | 106810.04 | 213629.07 | 448267.11 | 640905.16 | 854543.2 |
| 17 | Li | 20575.18 | 41163.23 | 82339.31 | 123515.40 | 164691.48 |
| 18 | Sb | 170054.93 | 300652.96 | 601849.00 | 903045.05 | 1204241.09 |
| 19 | Ba | 402921.67 | 808388.46 | 1619322.02 | 2430255.59 | 3241189.15 |
| 20 | Cu | 1140313.06 | 2281970.87 | 4565286.49 | 6848602.11 | 9131917.73 |
| 21 | Mo | 908258.34 | 1996842.57 | 3994011.03 | 5991179.49 | 7988347.95 |
| 22 | Sn | 947049.13 | 1969110.49 | 3921233.21 | 5883355.93 | 7845478.65 |
| 23 | Cr | 1861397.59 | 3723096.40 | 7446494.00 | 11169891.61 | 14893289.21 |

Table 11: Linearity Results of Elemental Impurities

| S. No. | Elements | Correlation Co efficient | Slope |
| --- | --- | --- | --- |
| 1 | Cd | 0.9997 | 2161.47 |
| 2 | Pb | 0.9999 | 22129.11 |
| 3 | As | 0.9998 | 396.23 |
| 4 | Hg | 0.9999 | 5050.00 |
| 5 | Co | 1.0000 | 9303.23 |
| 6 | V | 0.9995 | 3871.45 |
| 7 | Ni | 0.9998 | 2437.33 |
| 8 | Tl | 1.0000 | 54630.22 |
| 9 | Au | 0.9997 | 22580.02 |
| 10 | Pd | 1.0000 | 7762.41 |
| 11 | Ir | 1.0000 | 46124.25 |
| 12 | Rh | 1.0000 | 26852.13 |
| 13 | Ru | 1.0000 | 12281.85 |
| 14 | Se | 0.9999 | 28.18 |
| 15 | Ag | 0.9994 | 14272.61 |
| 16 | Pt | 0.9997 | 21502.34 |
| 17 | Li | 1.0000 | 748.57 |
| 18 | Sb | 0.9999 | 2511.15 |
| 19 | Ba | 1.0000 | 5786.07 |
| 20 | Cu | 1.0000 | 7609.50 |
| 21 | Mo | 0.9999 | 6651.89 |
| 22 | Sn | 1.0000 | 3268.16 |
| 23 | Cr | 1.0000 | 3384.81 |

Figure 01: Linearity of Cadmium

Figure 02: Linearity of Lead

Figure 03: Linearity of Arsenic

Figure 04: Linearity of Mercury

Figure 05: Linearity of Cobalt

Figure 06: Linearity of Vanadium

Figure 07: Linearity of Nickel

Figure 08: Linearity of Thallium

Figure 09: Linearity of Gold

Precision

- Six spiked (as per specification) individual samples were prepared as procedure mentioned in accuracy and analysed

- % RSD for six determinations NMT 20%

Table 12: Method Precision results in intensity (cps)

| S. No. | Elements | Method Precision Sample -01 | Method Precision Sample -02 | Method Precision Sample -03 | Method Precision Sample -04 | Method Precision Sample -05 | Method Precision Sample -06 | Sample Blank |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Cd | 2143.88 | 2189.34 | 2203.23 | 2133.45 | 2220.34 | 2122.45 | 2143.88 |
| 2 | Pb | 24151.34 | 23123.12 | 23273.03 | 23182.12 | 23819.99 | 23182.82 | 24151.34 |
| 3 | As | 1211.21 | 1219.55 | 1250.67 | 1256.45 | 1226.67 | 1193.56 | 1211.21 |
| 4 | Hg | 30199.13 | 30543.34 | 30356.11 | 30358.57 | 30835.21 | 30135.34 | 30199.13 |
| 5 | Co | 93024.64 | 93432.12 | 94737.11 | 93839.34 | 92834.22 | 93459.67 | 93024.64 |
| 6 | V | 77394.98 | 77436.34 | 77384.04 | 77490.34 | 77392.56 | 77284.09 | 77394.98 |
| 7 | Ni | 97850.83 | 97958.23 | 98075.34 | 97827.34 | 97947.11 | 97934.12 | 97850.83 |
| 8 | Tl | 87351.48 | 87457.23 | 87869.67 | 87348.11 | 87396.56 | 87436.34 | 87351.48 |
| 9 | Au | 449925.22 | 448938.23 | 448637.82 | 448362.02 | 447282.12 | 449352.12 | 449925.22 |
| 10 | Pd | 155142.35 | 154363.93 | 154363.39 | 155274.98 | 156869.85 | 155475.49 | 155142.35 |
| 11 | Ir | 923654.18 | 924728.23 | 925729.23 | 923657.45 | 923478.98 | 925474.56 | 923654.18 |
| 12 | Rh | 536847.38 | 537858.32 | 536566.67 | 535465.56 | 536747.98 | 535748.34 | 536847.38 |
| 13 | Ru | 245534.30 | 246898.89 | 245678.09 | 246878.09 | 245788.98 | 246887.09 | 245534.30 |
| 14 | Se | 855.54 | 856.12 | 854.34 | 855.98 | 853.09 | 855.12 | 855.54 |
| 15 | Ag | 425781.53 | 426585.05 | 425374.45 | 426585.67 | 425274.45 | 425745.54 | 425781.53 |
| 16 | Pt | 427263.54 | 427866.11 | 426748.67 | 427949.45 | 426273.45 | 427449.45 | 427263.54 |
| 17 | Li | 78939.09 | 79038.11 | 78892.02 | 78924.23 | 78839.09 | 79032.34 | 78939.09 |
| 18 | Sb | 623107.13 | 625839.98 | 631838.56 | 643939.34 | 632849.45 | 634546.56 | 623107.13 |
| 19 | Ba | 1620326.70 | 1647272.88 | 1638589.65 | 1645950.65 | 1634363.44 | 1648393.66 | 1620326.70 |
| 20 | Cu | 4475647.49 | 4467282.34 | 4457575.45 | 4469897.56 | 4448356.87 | 4454564.87 | 4475647.49 |
| 21 | Mo | 3794057.54 | 3804888.22 | 3818384.22 | 3797474.31 | 3802828.31 | 3794747.21 | 3794057.54 |
| 22 | Sn | 3924024.00 | 3942628.22 | 3917389.09 | 3956474.83 | 3962526.21 | 3927747.01 | 3924024.00 |
| 23 | Cr | 7326509.65 | 7306766.43 | 7296767.98 | 7302883.34 | 7284474.36 | 7326256.01 | 7326509.65 |
| 24 | Sample Weight(g) | 0.05004 | 0.05009 | 0.05003 | 0.05005 | 0.05009 | 0.05003 |  |

Table 13: Accuracy Results for Cadmium

| S. No | Sample name | Amount added | Amount Recovered | % Recovery | Average % Recovery |
| --- | --- | --- | --- | --- | --- |
| 1 | LOQ Preparation -01 | 0.125 | 0.121 | 96.80 | 97.60 |
| 2 | LOQ Preparation -02 |  | 0.122 | 97.60 |  |
| 3 | LOQ Preparation -03 |  | 0.123 | 98.40 |  |
| 4 | 50% Preparation -01 | 0.250 | 0.245 | 98.00 | 100.00 |
| 5 | 50% Preparation -02 |  | 0.255 | 102.00 |  |
| 6 | 50% Preparation -03 |  | 0.250 | 100.00 |  |
| 7 | 100% Preparation -01 | 0.500 | 0.494 | 98.80 | 99.87 |
| 8 | 100% Preparation -02 |  | 0.504 | 100.80 |  |
| 9 | 100% Preparation -03 |  | 0.507 | 101.40 |  |
| 10 | 100% Preparation -04 |  | 0.491 | 98.20 |  |
| 11 | 100% Preparation -05 |  | 0.511 | 102.20 |  |
| 12 | 100% Preparation -06 |  | 0.489 | 97.80 |  |
| 13 | 150% Preparation -01 | 0.750 | 0.743 | 99.07 | 99.02 |
| 14 | 150% Preparation -02 |  | 0.717 | 95.60 |  |
| 15 | 150% Preparation -03 |  | 0.768 | 102.40 |  |

Table 14: Accuracy Results for Lead

| S. No | Sample name | Amount added | Amount Recovered | % Recovery | Average % Recovery |
| --- | --- | --- | --- | --- | --- |
| 1 | LOQ Preparation -01 | 0.125 | 0.127 | 101.60 | 100.80 |
| 2 | LOQ Preparation -02 |  | 0.126 | 100.80 |  |
| 3 | LOQ Preparation -03 |  | 0.125 | 100.00 |  |
| 4 | 50% Preparation -01 | 0.250 | 0.244 | 97.60 | 101.87 |
| 5 | 50% Preparation -02 |  | 0.261 | 104.40 |  |
| 6 | 50% Preparation -03 |  | 0.259 | 103.60 |  |
| 7 | 100% Preparation -01 | 0.500 | 0.533 | 106.60 | 103.40 |
| 8 | 100% Preparation -02 |  | 0.509 | 101.80 |  |
| 9 | 100% Preparation -03 |  | 0.513 | 102.60 |  |
| 10 | 100% Preparation -04 |  | 0.511 | 102.20 |  |
| 11 | 100% Preparation -05 |  | 0.525 | 105.00 |  |
| 12 | 100% Preparation -06 |  | 0.511 | 102.20 |  |
| 13 | 150% Preparation -01 | 0.750 | 0.772 | 102.93 | 102.27 |
| 14 | 150% Preparation -02 |  | 0.754 | 100.53 |  |
| 15 | 150% Preparation -03 |  | 0.775 | 103.33 |  |

Table 15: Accuracy Results for Arsenic

| S. No | Sample name | Amount added | Amount Recovered | % Recovery | Average % Recovery |
| --- | --- | --- | --- | --- | --- |
| 1 | LOQ Preparation -01 | 0.375 | 0.378 | 100.80 | 101.69 |
| 2 | LOQ Preparation -02 |  | 0.382 | 101.87 |  |
| 3 | LOQ Preparation -03 |  | 0.384 | 102.40 |  |
| 4 | 50% Preparation -01 | 0.750 | 0.743 | 99.07 | 99.60 |
| 5 | 50% Preparation -02 |  | 0.747 | 99.60 |  |
| 6 | 50% Preparation -03 |  | 0.751 | 100.13 |  |
| 7 | 100% Preparation -01 | 1.500 | 1.512 | 100.80 | 102.04 |
| 8 | 100% Preparation -02 |  | 1.521 | 101.40 |  |
| 9 | 100% Preparation -03 |  | 1.562 | 104.13 |  |
| 10 | 100% Preparation -04 |  | 1.569 | 104.60 |  |
| 11 | 100% Preparation -05 |  | 1.530 | 102.00 |  |
| 12 | 100% Preparation -06 |  | 1.490 | 99.33 |  |
| 13 | 150% Preparation -01 | 2.250 | 2.284 | 101.51 | 103.14 |
| 14 | 150% Preparation -02 |  | 2.325 | 103.33 |  |
| 15 | 150% Preparation -03 |  | 2.353 | 104.58 |  |

Table 16: Accuracy Results for Mercury

| S. No | Sample name | Amount added | Amount Recovered | % Recovery | Average % Recovery |
| --- | --- | --- | --- | --- | --- |
| 1 | LOQ Preparation -01 | 0.750 | 97.73 | 97.73 | 98.27 |
| 2 | LOQ Preparation -02 |  | 98.93 | 98.93 |  |
| 3 | LOQ Preparation -03 |  | 98.13 | 98.13 |  |
| 4 | 50% Preparation -01 | 1.500 | 98.73 | 98.73 | 99.62 |
| 5 | 50% Preparation -02 |  | 99.60 | 99.60 |  |
| 6 | 50% Preparation -03 |  | 100.53 | 100.53 |  |
| 7 | 100% Preparation -01 | 3.000 | 99.30 | 99.30 | 99.96 |
| 8 | 100% Preparation -02 |  | 100.33 | 100.33 |  |
| 9 | 100% Preparation -03 |  | 99.87 | 99.87 |  |
| 10 | 100% Preparation -04 |  | 99.83 | 99.83 |  |
| 11 | 100% Preparation -05 |  | 101.30 | 101.30 |  |
| 12 | 100% Preparation -06 |  | 99.13 | 99.13 |  |
| 13 | 150% Preparation -01 | 4.500 | 99.51 | 99.51 | 99.76 |
| 14 | 150% Preparation -02 |  | 99.98 | 99.98 |  |

Limit of quantification (LOQ Precision)

- LOQ Level (25% from spec level) standards injected six times to determine precision

- % RSD of Intensity NMT 20%.

Table 17: LOQ results in intensity (cps)

| S. No | Elements | LOQ – 01 | LOQ – 02 | LOQ - 03 | LOQ - 04 | LOQ - 05 | LOQ - 06 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Cd | 532.46 | 530.90 | 532.32 | 532.65 | 530.36 | 528.78 |
| 2 | Pb | 5439.61 | 5431.11 | 5532.11 | 5692.00 | 5431.20 | 5461.20 |
| 3 | As | 280.52 | 283.02 | 283.90 | 282.79 | 281.80 | 282.11 |
| 4 | Hg | 7548.30 | 7616.03 | 7551.09 | 7517.16 | 7591.01 | 7663.12 |
| 5 | Co | 23250.17 | 23437.86 | 23436.44 | 23354.99 | 23144.33 | 23667.37 |
| 6 | V | 18694.79 | 18666.77 | 18592.51 | 18665.89 | 18546.00 | 18766.87 |
| 7 | Ni | 24313.69 | 24442.97 | 24431.30 | 24335.74 | 24514.53 | 24835.41 |
| 8 | Tl | 21834.27 | 21826.55 | 21772.06 | 21834.65 | 21661.98 | 21635.87 |
| 9 | Au | 112515.05 | 113379.45 | 112681.32 | 113408.19 | 114401.56 | 114301.43 |
| 10 | Pd | 38806.55 | 38468.46 | 38419.62 | 37457.44 | 38459.42 | 38460.66 |
| 11 | Ir | 228069.29 | 229459.25 | 229585.47 | 227386.22 | 229478.46 | 231478.59 |
| 12 | Rh | 133917.13 | 132840.56 | 133847.71 | 131940.67 | 133947.64 | 134051.82 |
| 13 | Ru | 61400.07 | 61522.03 | 61524.64 | 62424.14 | 61525.03 | 62445.81 |
| 14 | Se | 206.11 | 206.29 | 207.30 | 207.00 | 207.78 | 206.00 |
| 15 | Ag | 106367.17 | 105374.60 | 104247.58 | 103389.03 | 103377.70 | 103367.20 |
| 16 | Pt | 106810.04 | 105475.50 | 104840.27 | 105364.36 | 106368.59 | 106667.35 |
| 17 | Li | 20575.18 | 20662.09 | 20521.36 | 20633.55 | 20661.46 | 20454.69 |
| 18 | Sb | 150054.93 | 149834.85 | 150820.22 | 151824.12 | 152205.26 | 155125.40 |
| 19 | Ba | 402921.67 | 404100.98 | 401131.19 | 403119.30 | 401212.31 | 403133.34 |
| 20 | Cu | 1140313.06 | 1145028.88 | 1155403.18 | 1152288.82 | 1145103.68 | 1147442.68 |
| 21 | Mo | 998258.34 | 998037.23 | 983307.56 | 984320.75 | 981908.56 | 982442.23 |
| 22 | Sn | 978049.13 | 979533.77 | 973540.89 | 971632.02 | 981533.35 | 985665.11 |
| 23 | Cr | 1861397.59 | 1863233.88 | 1853466.12 | 1864246.23 | 1853235.79 | 1858686.57 |

Limit of Detection

Table 18: LOD level results (cps)

| S. No | Element s | LOD – 01(cps) | LOD – 02(cps) | LOD – 03(cps) | Average (cps) | LOD(pp b) | LOD(pp m) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Cd | 171.98 | 171.55 | 172.77 | 172.10 | 0.08 | 0.04 |
| 2 | Pb | 1723.90 | 1750.22 | 1728.11 | 1734.08 | 0.078 | 0.039 |
| 3 | As | 75.61 | 74.91 | 77.54 | 76.02 | 0.192 | 0.096 |
| 4 | Hg | 2475.99 | 2405.91 | 2471.36 | 2451.09 | 0.485 | 0.243 |
| 5 | Co | 7666.14 | 7634.21 | 7675.52 | 7658.62 | 0.823 | 0.412 |
| 6 | V | 5699.24 | 5703.41 | 5700.75 | 5701.13 | 1.473 | 0.737 |
| 7 | Ni | 8002.00 | 8026.33 | 8046.55 | 8024.96 | 3.293 | 1.647 |
| 8 | Tl | 7197.56 | 7199.53 | 7197.65 | 7198.25 | 0.132 | 0.066 |
| 9 | Au | 37085.82 | 36509.45 | 36206.87 | 36600.71 | 1.621 | 0.811 |
| 10 | Pd | 12801.69 | 12752.27 | 12750.38 | 12768.11 | 1.645 | 0.823 |
| 11 | Ir | 73193.99 | 73265.58 | 72559.47 | 73006.35 | 1.583 | 0.792 |
| 12 | Rh | 43914.15 | 43856.62 | 43149.79 | 43640.19 | 1.625 | 0.813 |
| 13 | Ru | 20254.59 | 20124.12 | 20113.58 | 20164.10 | 1.642 | 0.821 |
| 14 | Se | 62.21 | 62.89 | 64.12 | 63.07 | 2.238 | 1.119 |
| 15 | Ag | 35041.11 | 35576.70 | 35778.76 | 35465.52 | 2.485 | 1.243 |
| 16 | Pt | 35241.30 | 35788.34 | 35749.88 | 35593.17 | 1.655 | 0.828 |
| 17 | Li | 6781.19 | 6770.22 | 6780.90 | 6777.44 | 9.054 | 4.527 |
| 18 | Sb | 49154.26 | 49142.00 | 49215.36 | 49170.54 | 19.581 | 9.791 |
| 19 | Ba | 131258.93 | 132212.84 | 133213.01 | 132228.26 | 22.853 | 11.427 |
| 20 | Cu | 375402.33 | 374202.22 | 373505.19 | 374369.91 | 49.198 | 24.599 |
| 21 | Mo | 329206.91 | 325332.17 | 324523.05 | 326354.04 | 49.062 | 24.531 |
| 22 | Sn | 320738.02 | 323644.41 | 321976.86 | 322119.76 | 98.563 | 49.282 |
| 23 | Cr | 614059.39 | 613557.55 | 614486.22 | 614034.39 | 181.409 | 90.705 |

SUMMARY AND CONNCLUSION

In this work, a new ICP-MS approach for listed medications with active pharmaceutical ingredients was created.

- Clarithromycin: The created approaches were validated in a various types of parameters, including

- Linearity / SST

- Precision

- Accuracy

- Limit of detection

- Limit of quantification

- Specificity

- Ruggedness

The revised regulations for elemental impurities offer enhanced data to detect specific elemental impurities and assess their possible impact for patient safety. The ideal operating parameters for the ICP-MS technique, employed in the analysis of elemental impurities, were effectively established by multivariate optimization. A method utilizing ICP-MS for quantifying elemental impurities in active pharmaceutical ingredients has been developed in compliance with the new USP Section 232 and Limits-233 procedures for Elemental Impurities. A strong correlation was identified in the experimental values, with a minimum value of 0.99. Validation demonstrated that all elements exhibited recovery rates ranging from 70% to 150% and a relative standard deviation (RSD) within 20% for precision (repeatability), thereby meeting the USP standards. The method's ruggedness, indicated by a % RSD of less than 25, illustrates its capacity for reproducibility across numerous days. The LOD and LOQ values from the developed approach are optimally aligned for the quantification of elemental contaminants in accordance with USP target limits. Trace levels of elemental impurities including Iridium (Ir), Barium (Ba), Cadmium (Cd), Arsenic (As), Platinum (Pt), Mercury (Hg), Cobalt (Co), Nickel (Ni), Gold (Au), Vanadium (V), Palladium (Pd), Rhodium (Rh), Lithium (Li), Selenium (Se), Silver (Ag), Ruthenium (Ru), Molybdenum (Mo), Antimony (Sb), Copper (Cu), Tin (Sn), Chromium (Cr), Lead (Pb), and Thallium (Tl) in a single analysis. The methodology was developed in compliance with ICH- Q3D Option 1. The validation results have been presented and organized in tables to summarize the method's performance. An efficient ICP-MS method was developed for the concurrent analysis of 23 elements. It can simultaneously and accurately detect 23 things. The instrument's KED mode mitigates interference from oxides and polyatomic ions, resulting in a reduced detection time for each sample.

The Advantages of the Developed ICP-MS Method:

- No difficult extraction methods are required.

- Simultaneous examination of several elements in a single run

No internal standard is necessary

- The required running time is significantly reduced.

- Suitable for the examination of Clarithromycin API

- Economically efficient

Hence, the developed ICP-MS technique for Clarithromycin API, being rapid, simple, precise, specific, linear, accurate, and rugged. It can be applied for the routine analysis of selected.

REFERENCES

- C. Ludke, E. Hoffmann, J. Skole, and S. Artelt. Particle analysis of car exhaust by ETV- ICP-MS. Fresenius Journal of Analytical Chemistry 1996; 355: 261–263.

- C. Maqueda, E. Morillo. Determination of calcium by atomic absorption spectrometry in samples dissolved by acid mixtures. Fresenius Journal Analytical Chemistry 1990; 338: 253-254.

- D.M. Wayne. Direct determination of trace noble metals (palladium, platinum and rhodium) in automobile catalysts by glow discharge mass spectrometry. Journal of Analytical Atomic Spectrometry 1997; 12: 1195–1202.

- D.S.Strand, D.Kim, and D.A.Peura. 25 Years of proton pump inhibitors: a comprehensive review. Gut Liver 2017; 11: 27–37.

- F.Vanhaecke , M.Resano, J.Koch, K.McIntosh, and D.Gunther. Femtosecond laser ablation-ICP-mass spectrometry analysis of a heavy metallic matrix: determination of platinum group metals and gold in lead fire-assay buttons as a case study. Journal of Analytical Atomic Spectrometry 2010; 25:1259–1267.

- F.Vanhaecke, M.Resano, E.Garcia-Rui, L.Balcaen, K.R.Koch,and K.McIntosh. Laser ablation-inductively coupled plasma-dynamic reaction cell-mass spectrometry (LA- ICP-DRC-MS) for the determination of Pt, Pd and Rh in Pb buttons obtained by fire assay of platiniferous ores. Journal of Analytical Atomic Spectrometry 2004; 19: 632– 638.

- F.Vanhaecke, M.Resano, M.PrunedaLopez, and L.Moens. Determination of platinum and rhodium in environmental matrixes by solid sampling-electro thermal vaporization- inductively coupled plasma mass spectrometry. Analytical Chemistry. 2002; 74: 6040– 6048.

- G.Asimellis, N.Michos, I.Fasaki,and M.Kompitsas. Platinum group metals bulk analysis in automobile catalyst recyclingmaterial by laser-induced breakdown spectroscopy. Spectrochimica Acta Part B: atomic spectroscopy 2008; 63: 1338–1343.

- J.S.Barin, P.A.Mello, M.F.Mesko, F.A.Duarte,and E.M.M.Flores. Determination of elemental impurities in pharmaceutical products and related matrices by ICP-based methods: a review. Analytical and Bioanalytical Chemistry 2016; 408: 4547–4566.

- K.Sudhakar, M.Sujatha, S.R.Babu, P.Padmavathi, and P.P.Reddy. Serum calcium levels in patients with essential hypertension and their first degree relatives. Indian Journal Clinical Biochemistry 2004; 19: 21-23.

- M.Balcerzak. Methods for the determination of platinum group elements in environmental and biological materials a review. Critical Review Analytical Chemistry 2011; 41: 214–235.

- M.Resano, E.Garcia Ruiz, M.A.Belarra, F.Vanhaecke, and K.S.McIntosh. Solid sampling in the determination of precious metals at ultratrace levels. Trends in Analytical Chemistry 2007; 26: 385–395.

- M.Resano, E.Garciaruiz, K.S.McIntosh, and F.Vanhaecke. Laser ablation-inductively coupled plasma-dynamic reaction cell-mass spectrometry for the determination of platinum group metals and gold in NiS buttons obtained by fire assay of platiniferous ores. Journal of Analytical Atomic Spectrometry 2008; 23: 1599–1609.

- M.Resano, E.Garcia-Ruiz, K.S.McIntosh, J.Hinrichs, I.Deconinck,and F.Vanhaecke. Comparison of the solid sampling techniques laser ablation ICP-MS, glow discharge MS and spark-OES for the determination of platinum group metals in Pb buttons obtained by fire assay of platiniferous ores. Journal Analytical Atomic Spectrometry 2006; 21: 899–909.

- M.Resano, K.S.McIntosh,and F.Vanhaecke. Laser ablation-inductively coupled plasma mass spectrometry using a double-focusing sector field mass spectrometer of Mattauch Herzog geometry and an array detector for the determination of platinum group metals and gold in NiS buttons obtained by fire assay of platiniferous ores. Journal of Analytical Atomic Spectrometry 2012; 27: 165–173.

- N.Lewen. Preparation of pharmaceutical samples for elemental impurities analysis: some potential approaches. Spectroscopy 2016; 31: 36–43.

- P.Lucena, J.M.Vadillo, and J.J.Laserna. Mapping of platinum group metals in automotive exhaust three-way catalysts using laser-induced breakdown spectrometry. Analytical Chemistry 1999; 71: 4385–4391.

- R.K.Jalali, D.Rasaily. Generic drug and bioequivalence studies. Pharmaceutical Medicine and Translation clinical research 2018; 327–339.

- R.W.Baker. The determination of calcium in serum by flame photometry. Biochemical Journal 1955; 59: 566-571.

- S.Compernolle, D.Wambeke, I.De Raedt, K.Kimpe, and F.Vanhaecke. Direct determination of Pd, Pt and Rh in fire assay lead buttons by laser ablation-ICP-OES: automotive exhaust catalysts as an example. Journal of Analytical Atomic Spectrometry. 2011; 26: 1679–1684.

- S.Rauch, G.M.Morrison,and M.Moldovan. Scanning laser ablation-ICP-MS tracking of platinum group elements in urban particles. Science of the Total Environment 2002; 286: 243–251.

- Z.Chen, I.J.Griffin, Y.L.Kriseman, L.K.Liang, and S.A.Abrams. Inductively coupled plasma mass spectrometric analysis of calcium isotopes in human serum: a low-sample volume acid equilibration method. Clinical Chemistry 2003; 49: 2050-2055.